# Welding & Fabrication Technician

Sector: Construction & Building Trades | Level: Foundation | Duration: 3 Months

# Module 1: Safety, Health & Tools

Q1. Welding karte samay sabse zaroori Personal Protective Equipment (PPE) kya hai?

A) Safety Goggles

B) Welding Helmet

C) Leather Gloves

D) Ear Plugs

Correct Answer: B

Explanation: Welding karte samay aankhon ko arc rays aur sparks se bachane ke liye welding helmet sabse zaroori hai.

Q2. Fire extinguisher ka sahi type kaun sa hai jo welding workshop mein electrical fire (Class C) ke liye use hota hai?

A) Water Type

B) Foam Type

C) CO2 Type

D) Dry Powder Type (ABC)

Correct Answer: C

Explanation: CO2 type extinguisher electrical fires (Class C) ke liye effective hai kyunki yeh residue nahi chhodta aur non-conductive hota hai.

Q3. Ek welder ko apne working area mein proper ventilation kyu maintain karna chahiye?

A) Taaki welding fumes se bacha ja sake

B) Taaki workshop thanda rahe

C) Taaki visibility achhi rahe

D) Taaki kaam jaldi ho

Correct Answer: A

Explanation: Proper ventilation welding fumes ko remove karta hai jo health ke liye harmful ho sakte hain.

Q4. Commonly use hone wala measuring tool jo metal sheets ki length aur width check karne ke liye kaam aata hai?

A) Vernier Caliper

B) Micrometer

C) Steel Tape Measure

D) Try Square

Correct Answer: C

Explanation: Steel tape measure lambi measurements aur general layout ke liye sabse common aur convenient tool hai.

Q5. Chhote aur intricate measurements jaise plate thickness ya rod diameter ke liye kaun sa tool best hai?

A) Steel Rule

B) Vernier Caliper

C) Measuring Tape

D) Divider

Correct Answer: B

Explanation: Vernier caliper high precision measurements ke liye use hota hai, jo steel rule se zyada accurate hota hai.

Q6. Metal surface par marking karne ke liye kya use kiya jata hai?

A) Chisel

B) Hammer

C) Scriber

D) File

Correct Answer: C

Explanation: Scriber ek sharp pointed tool hai jo metal surface par clear lines mark karne ke liye use hota hai.

Q7. Kisi metal piece ko support aur hold karne ke liye konsa tool use hota hai jab uspar kaam kiya ja raha ho (jaise filing ya cutting)?

A) C-Clamp

B) Welding Clamp

C) Bench Vice

D) Pliers

Correct Answer: C

Explanation: Bench vice ek sturdy clamping device hai jo workpieces ko securely hold karta hai.

Q8. Electrical shock se bachne ke liye kya precaution leni chahiye?

A) Wet gloves pehenna

B) Damaged cables se kaam karna

C) Properly insulated tools aur cables use karna

D) Nange paer kaam karna

Correct Answer: C

Explanation: Properly insulated tools aur cables electrical shock ke risk ko kam karte hain.

Q9. Angle grinder use karte samay kya precaution leni chahiye?

A) Safety glasses nahi pehenna

B) Loose kapde pehenna

C) Workpiece ko securely clamp karna

D) Guard ko remove karna

Correct Answer: C

Explanation: Workpiece ko securely clamp karne se grinder slip nahi hota aur operator safe rehta hai.

Q10. Welding fumes mein kaun se harmful substances ho sakte hain?

A) Oxygen aur Nitrogen

B) Carbon Monoxide aur Ozone

C) Water Vapor aur Argon

D) Helium aur Neon

Correct Answer: B

Explanation: Welding fumes mein carbon monoxide, ozone, aur various metal oxides jaise harmful gases ho sakte hain.

Q11. Kisi bhi power tool ko use karne se pehle kya check karna zaroori hai?

A) Tool ka colour

B) Tool ka weight

C) Cable, plug aur switch ki condition

D) Tool ki manufacturing date

Correct Answer: C

Explanation: Damaged cables, plugs ya switches electrical hazards create kar sakte hain. Always check before use.

Q12. Welding arc se nikalne wali UV rays se bachne ke liye kya pehenna chahiye?

A) Normal sunglasses

B) Clear safety glasses

C) Welding helmet with proper shade lens

D) Dust mask

Correct Answer: C

Explanation: Welding helmet ka shade lens UV rays ko block karta hai aur aankhon ko damage se bachata hai.

Q13. Hot metal ko handle karne ke liye kaun se gloves use karne chahiye?

A) Cotton gloves

B) Rubber gloves

C) Leather welding gloves

D) Disposable plastic gloves

Correct Answer: C

Explanation: Leather welding gloves heat resistant hote hain aur hot metal se protection provide karte hain.

Q14. Workshop mein oil spills ko immediately saaf karna kyu zaroori hai?

A) Taaki workshop clean lage

B) Taaki fire hazard na ho

C) Taaki koi slip hokar gire nahi

D) B aur C dono

Correct Answer: D

Explanation: Oil spills se fire hazard aur slipping hazard dono ho sakte hain, isliye unhe turant saaf karna chahiye.

Q15. Try Square ka primary use kya hai?

A) Measurements lena

B) Angles check karna aur mark karna (90 degree)

C) Holes drill karna

D) Metal cut karna

Correct Answer: B

Explanation: Try square metal pieces par 90 degree angles check karne aur mark karne ke liye use hota hai.

Q16. Ek welder ko apne kapde kaise pehenne chahiye?

A) Loose aur synthetic fabric

B) Tight aur synthetic fabric

C) Flame-resistant cotton ya leather, sleeves rolled down

D) Shorts aur T-shirt

Correct Answer: C

Explanation: Flame-resistant kapde sparks aur heat se bachate hain, aur sleeves rolled down hone chahiye taaki skin exposed na ho.

Q17. Welding area mein kis cheez ko store nahi karna chahiye?

A) Welding electrodes

B) Flammable liquids jaise paint thinner

C) Metal scrap

D) Welding machine

Correct Answer: B

Explanation: Welding ke sparks aur heat se flammable liquids mein aag lag sakti hai, isliye unhe welding area se door rakhna chahiye.

# Module 2: Basic Welding Processes (SMAW)

Q18. SMAW ka full form kya hai?

A) Shielded Metal Arc Welding

B) Solid Metal Arc Welding

C) Soft Metal Arc Welding

D) Strong Metal Arc Welding

Correct Answer: A

Explanation: SMAW ko 'Stick Welding' bhi kehte hain, aur iska full form Shielded Metal Arc Welding hai.

Q19. SMAW mein arc ko maintain karne ke liye kya zaroori hai?

A) Constant electrode movement

B) Proper arc length

C) Low amperage

D) Thick workpiece

Correct Answer: B

Explanation: Ek stable aur consistent arc ke liye proper arc length maintain karna bahut zaroori hai.

Q20. E6013 electrode kis type ke material ke liye commonly use hota hai?

A) Stainless Steel

B) Cast Iron

C) Mild Steel

D) Aluminum

Correct Answer: C

Explanation: E6013 general purpose mild steel welding ke liye bahut popular electrode hai, jo easy-to-use hota hai.

Q21. Welding current ki unit kya hai?

A) Volts

B) Ohms

C) Amperes

D) Watts

Correct Answer: C

Explanation: Welding current ko Amperes (Amps) mein measure kiya jata hai.

Q22. AC welding machine ka ek fayda kya hai?

A) Smoother arc

B) Arc blow ko kam karta hai

C) Thinner materials ke liye best

D) Deep penetration deta hai

Correct Answer: B

Explanation: AC welding machines arc blow ko kam karne mein madad karte hain, jo DC welding mein ek common problem hai.

Q23. DCEN (Direct Current Electrode Negative) polarity mein heat kahan zyada generate hoti hai?

A) Electrode par

B) Workpiece par

C) Dono par equally

D) Welding machine mein

Correct Answer: B

Explanation: DCEN mein, current electrode se workpiece ki taraf flow karta hai, jisse workpiece par approximately 2/3 heat generate hoti hai.

Q24. SMAW mein flux ka kya kaam hai?

A) Arc ko ignite karna

B) Molten weld pool ko atmospheric contamination se bachana

C) Welding current ko control karna

D) Electrode ko thanda karna

Correct Answer: B

Explanation: Flux melt hokar shielding gas aur slag banata hai, jo molten weld pool ko oxygen aur nitrogen se bachata hai.

Q25. Short arc length hone se kya ho sakta hai?

A) Poor penetration

B) Excessive spatter

C) Arc extinguishing (sticking)

D) Wider bead

Correct Answer: C

Explanation: Bahut short arc length hone par electrode workpiece se touch kar sakta hai aur arc bujh sakta hai (sticking).

Q26. Long arc length hone se kya ho sakta hai?

A) Deep penetration

B) Narrow bead

C) Increased spatter aur poor shielding

D) Smooth weld bead

Correct Answer: C

Explanation: Long arc length se shielding gas dissolve ho jata hai, jisse spatter badhta hai aur weld quality kharab hoti hai.

Q27. E7018 electrode ki kya khas baat hai?

A) Fast freeze electrode hai

B) Low hydrogen electrode hai

C) Non-ferrous metals ke liye hai

D) AC power ke liye hi hai

Correct Answer: B

Explanation: E7018 ek low hydrogen electrode hai, jo high strength welds aur crack resistance ke liye use hota hai.

Q28. Welding ke baad slag ko remove karna kyu zaroori hai?

A) Taaki weld thanda ho sake

B) Taaki next pass kiya ja sake aur weld inspection ho sake

C) Taaki weld shine kare

D) Isse weld strength badhti hai

Correct Answer: B

Explanation: Slag ko remove karne se agla weld pass properly deposit ho pata hai aur weld defects ko inspect kiya ja sakta hai.

Q29. Welding rod ko kis angle par hold karna chahiye?

A) 90 degrees workpiece se

B) 45 degrees workpiece se

C) 70-80 degrees workpiece se

D) Parallel workpiece ke

Correct Answer: C

Explanation: Typically, electrode ko 70-80 degree angle par hold kiya jata hai taaki proper penetration aur bead shape mil sake.

Q30. Ek welding machine mein 'Duty Cycle' ka kya matlab hai?

A) Machine ka total weight

B) Kitni der tak machine continuous weld kar sakti hai 10-minute cycle mein

C) Machine ka power consumption

D) Machine ki maximum current setting

Correct Answer: B

Explanation: Duty cycle percentage mein batata hai ki ek machine 10-minute period mein kitni der tak rated output current par weld kar sakti hai overheat hue bina.

Q31. Arc Welding mein 'Arc Blow' kya hai?

A) Arc ka bahut zyada heat produce karna

B) Magnetic forces ke karan arc ka deflected hona

C) Arc ka bahut chhota hona

D) Arc ka baar baar bujh jana

Correct Answer: B

Explanation: Arc blow magnetic fields ke karan arc ka unstable ya deflected ho jana hai, jo DC welding mein common hai.

Q32. SMAW mein kaun sa factor penetration ko directly affect karta hai?

A) Electrode coating type

B) Welding current (Amperage)

C) Arc length

D) Travel speed

Correct Answer: B

Explanation: Higher amperage zyada heat produce karta hai, jisse deeper penetration milti hai.

Q33. Rutile electrodes (jaise E6013) ki kya khas baat hai?

A) Very deep penetration

B) Smooth arc aur easy slag removal

C) High strength welds only

D) Only for overhead welding

Correct Answer: B

Explanation: Rutile electrodes smooth arc, easy slag removal, aur good bead appearance dete hain, making them user-friendly.

Q34. Welding machine ke earth clamp ko kahan attach karna chahiye?

A) Welder ke kapdon par

B) Workpiece ke jitna ho sake utna kareeb

C) Welding machine par

D) Kisi bhi metal structure par

Correct Answer: B

Explanation: Earth clamp ko workpiece ke jitna ho sake utna kareeb attach karne se current path short hota hai aur arc stability badhti hai.

# Module 3: Material Identification & Preparation

Q35. Mild steel ko kaise identify kar sakte hain?

A) Ismein magnet nahi lagta

B) Iska surface bahut rough hota hai

C) Ismein magnet lagta hai aur yeh easily weld ho jata hai

D) Yeh bahut lightweight hota hai

Correct Answer: C

Explanation: Mild steel ferromagnetic hota hai (magnet lagta hai) aur iski weldability achhi hoti hai.

Q36. Stainless steel ki ek khas property kya hai?

A) Rust lagta hai

B) Corrosion resistant hota hai

C) Bohut soft hota hai

D) Easily bend nahi hota

Correct Answer: B

Explanation: Stainless steel mein chromium hota hai jo use corrosion resistant banata hai.

Q37. Welding se pehle metal surfaces ko clean karna kyu zaroori hai?

A) Taaki weld achha dikhe

B) Taaki weld defects jaise porosity aur poor fusion se bacha ja sake

C) Taaki welding faster ho

D) Taaki electrode melt na ho

Correct Answer: B

Explanation: Oil, grease, rust ya paint jaise contaminants weld quality ko kharab karte hain aur defects create karte hain.

Q38. Butt joint mein do metal pieces ko kaise rakha jata hai?

A) Ek ke upar ek

B) Ek doosre ke edge to edge

C) 90 degree angle par

D) Parallel lekin thoda gap ke saath

Correct Answer: B

Explanation: Butt joint mein do metal pieces ko same plane mein edge to edge rakha jata hai aur unke edges ko weld kiya jata hai.

Q39. Lap joint mein metal pieces ko kaise place kiya jata hai?

A) Edges ek doosre se touch karte hue

B) Ek piece doosre ke upar overlap karta hua

C) 90 degree par ek doosre se milte hue

D) Gap ke saath

Correct Answer: B

Explanation: Lap joint mein ek metal piece doosre ke upar overlap karta hai aur edges par weld kiya jata hai.

Q40. Thick plates ko full penetration weld karne ke liye kaun sa edge preparation commonly use hota hai?

A) Square edge

B) Bevel edge

C) Rounded edge

D) Flat edge

Correct Answer: B

Explanation: Bevel edge preparation thick plates mein deep penetration aur strong weld ke liye joint design mein space provide karta hai.

Q41. Oxy-fuel cutting mein kaun si gases use hoti hain?

A) Argon aur Helium

B) Oxygen aur Acetylene

C) Nitrogen aur Hydrogen

D) CO2 aur Oxygen

Correct Answer: B

Explanation: Oxy-fuel cutting mein oxygen aur fuel gas (jaise acetylene) ka mixture metal ko preheat aur cut karta hai.

Q42. Grinding wheel se metal ko cut karte samay kya precaution leni chahiye?

A) Loose kapde pehenna

B) Safety glasses aur face shield pehenna

C) High speed par grind karna

D) Workpiece ko haath se pakadna

Correct Answer: B

Explanation: Grinding se sparks aur metal particles udte hain, isliye aankhon aur chehre ko protect karna zaroori hai.

Q43. T-joint mein do metal pieces kis angle par milte hain?

A) 180 degrees

B) 45 degrees

C) 90 degrees

D) 30 degrees

Correct Answer: C

Explanation: T-joint mein ek metal piece doosre ke surface par perpendicular (90 degrees) join hota hai.

Q44. Welding se pehle joint edges ko 'root gap' kyu diya jata hai?

A) Taaki weld thanda ho sake

B) Taaki electrode easily enter ho sake aur full penetration mil sake

C) Taaki weld zyada strong bane

D) Taaki metal kam use ho

Correct Answer: B

Explanation: Root gap proper penetration aur fusion ke liye zaroori hai, khas kar butt joints mein.

Q45. Metal ko hacksaw se cut karte samay kya sahi technique hai?

A) Fast aur heavy pressure

B) Slow aur light pressure

C) Forward stroke par pressure aur return stroke par no pressure

D) Backward stroke par pressure aur forward stroke par no pressure

Correct Answer: C

Explanation: Hacksaw blades cutting action forward stroke par karte hain, isliye pressure forward stroke par apply karna chahiye.

Q46. Cast iron ki welding mein kya dikkat aati hai?

A) Yeh bahut soft hota hai

B) Ismein high carbon content hota hai, jisse yeh brittle hota hai aur crack hone ka risk hota hai

C) Yeh easily melt nahi hota

D) Ismein magnet nahi lagta

Correct Answer: B

Explanation: Cast iron mein high carbon content use brittle banata hai, jisse welding ke dauran cracks ka risk badh jata hai. Pre-heating aur slow cooling zaroori hai.

Q47. Welding se pehle metal surface se rust remove karne ke liye kaun sa tool use hota hai?

A) Measuring tape

B) Wire brush ya grinder

C) Hammer

D) Scriber

Correct Answer: B

Explanation: Wire brush ya grinder se rust aur scale ko effectively remove kiya ja sakta hai.

Q48. Fillet weld kis joint type par commonly use hota hai?

A) Butt joint

B) Lap joint aur T-joint

C) Edge joint

D) Corner joint, jab edges flush ho

Correct Answer: B

Explanation: Fillet weld lap joints aur T-joints mein do surfaces ke beech ke angle mein banaya jata hai.

Q49. Ek thin metal sheet ko cut karne ke liye sabse suitable method kya hai?

A) Oxy-fuel cutting

B) Plasma cutting

C) Manual hacksaw ya Nibbler

D) Grinder with thick wheel

Correct Answer: C

Explanation: Thin sheets ke liye manual hacksaw ya nibbler zyada precise aur kam heat input wali method hai, distortion kam hoti hai.

Q50. Weld joint ki strength ko improve karne ke liye kya kiya ja sakta hai?

A) Less penetration

B) Proper joint preparation aur sufficient weld metal deposition

C) Fast travel speed

D) Low amperage

Correct Answer: B

Explanation: Achhi joint preparation aur proper amount mein weld metal deposit karne se joint ki strength badhti hai.

Q51. Metal surface par paint ya coating hone se welding par kya asar padta hai?

A) Weld zyada strong banta hai

B) Weld defects jaise porosity aur lack of fusion ho sakte hain

C) Welding process faster ho jata hai

D) Koi asar nahi padta

Correct Answer: B

Explanation: Paint aur coatings weld zone mein contaminants introduce karte hain, jisse porosity, lack of fusion aur fumes ki problems hoti hain.

# Module 4: Basic Fabrication Techniques

Q52. Layout marking ke liye kaun sa tool use hota hai taaki metal sheet par precise lines banayi ja sake?

A) Chisel

B) Punch

C) Scriber aur Steel Rule

D) Hammer

Correct Answer: C

Explanation: Scriber aur steel rule ka combination precise marking ke liye essential hai.

Q53. Metal sheets ko manually bend karne ke liye kya use hota hai?

A) Welding machine

B) Bench vice aur Hammer

C) Grinder

D) Drill machine

Correct Answer: B

Explanation: Chhote bends ke liye bench vice mein sheet ko clamp karke hammer se bend kiya ja sakta hai.

Q54. Burrs kya hote hain aur unhe kyu remove karna chahiye?

A) Yeh weld defects hain, unhe rehne dena chahiye

B) Yeh sharp metal edges hain, unhe safety aur fit-up ke liye remove karna chahiye

C) Yeh weld ko strong banate hain

D) Yeh sirf decorative hote hain

Correct Answer: B

Explanation: Burrs cutting ya grinding ke baad bache sharp edges hote hain, jo injuries cause kar sakte hain aur proper fit-up mein rukawat dal sakte hain.

Q55. Tack welding ka primary purpose kya hai?

A) Final weld banana

B) Components ko temporarily hold karna aur unki alignment maintain karna

C) Metal ko heat up karna

D) Weld defects ko cover karna

Correct Answer: B

Explanation: Tack welds chhote, temporary welds hote hain jo components ko position mein rakhte hain final welding se pehle.

Q56. Sheet metal ko straight line mein cut karne ke liye kaun sa manual tool use hota hai?

A) Chisel

B) Tin snips (Sheet metal shears)

C) Hacksaw

D) Drill

Correct Answer: B

Explanation: Tin snips ya sheet metal shears thin sheets ko straight aur curved lines mein cut karne ke liye designed hote hain.

Q57. Holes drill karne se pehle kya karna zaroori hai?

A) Directly drill karna shuru kar dena

B) Center punch se mark karna

C) Metal ko heat karna

D) Welding machine on karna

Correct Answer: B

Explanation: Center punch se mark karne se drill bit apni jagah par rehti hai aur slip nahi hoti.

Q58. Metal ko smooth finish dene ke liye kaun sa tool use hota hai?

A) Hammer

B) File

C) Chisel

D) Scriber

Correct Answer: B

Explanation: Files metal surfaces ko smooth karne, burrs remove karne aur shapes refine karne ke liye use hote hain.

Q59. Fitting up operations mein components ko align karne ke liye kya use hota hai?

A) Measuring tape

B) Squares, clamps aur levels

C) Welding electrodes

D) Grinding wheel

Correct Answer: B

Explanation: Squares, clamps, aur levels components ko correct position aur angle mein hold karne aur align karne mein help karte hain.

Q60. Distortion ko kam karne ke liye kya technique use hoti hai during welding?

A) Long continuous welds

B) Intermittent welding aur proper clamping

C) High heat input

D) Thinner electrodes

Correct Answer: B

Explanation: Intermittent welds (skip welding) aur strong clamping metal ko warp hone se bachate hain. Pre-setting aur balanced welding bhi helpful hai.

Q61. Grinding karte samay guard ka kya purpose hai?

A) Grinder ko balance karna

B) Sparks aur debris se operator ko protect karna

C) Grinding wheel ko thanda karna

D) Grinder ko speed dena

Correct Answer: B

Explanation: Guard operator ko flying sparks, debris aur wheel rupture se protect karta hai.

Q62. Drill bit ko workpiece par kyu press karna chahiye?

A) Taaki drill bit garam ho jaye

B) Taaki cutting action ho aur chips remove ho

C) Taaki hole bada bane

D) Taaki drill bit toot jaye

Correct Answer: B

Explanation: Proper pressure apply karne se drill bit material ko cut karti hai aur chips remove karti hai, jisse efficient drilling hoti hai.

Q63. Ek welder ko apne tools ko kaise maintain karna chahiye?

A) Unhe ganda aur unorganized rakhna

B) Regularly clean karna, inspect karna aur sharpen karna

C) Unhe kabhi saaf nahi karna

D) Dusre welders ke tools use karna

Correct Answer: B

Explanation: Regular maintenance se tools ki life badhti hai, performance achhi rehti hai aur safety maintain hoti hai.

Q64. Marking fluid (Layout dye) ka kya use hai fabrication mein?

A) Metal ko colour karna

B) Metal par temporary lines banana

C) Metal surface ko dark karna taaki scribed lines clearly dikhein

D) Metal ko protect karna

Correct Answer: C

Explanation: Layout dye metal surface ko blue ya black kar deta hai, jisse scriber se banayi gayi shiny lines clearly visible hoti hain.

Q65. Sheet metal bending ke liye 'bending brake' machine ka kya fayda hai?

A) Sirf gol bends banata hai

B) Accurate aur sharp bends banata hai

C) Sirf bahut thick metal bend karta hai

D) Welding ke liye use hota hai

Correct Answer: B

Explanation: Bending brake (ya press brake) machine sheet metal mein precise aur sharp angles banane ke liye ideal hai.

Q66. Pipes ko cut karne ke liye kaun sa tool use ho sakta hai?

A) Tin snips

B) Pipe cutter ya hacksaw

C) Bench vice

D) Scriber

Correct Answer: B

Explanation: Pipe cutter pipes ko clean aur straight cut karne ke liye designed hai, hacksaw bhi use ho sakti hai.

Q67. Welding fixture ka kya role hai fabrication mein?

A) Welding current control karna

B) Workpieces ko accurately position aur hold karna welding ke dauran

C) Welding fumes ko remove karna

D) Welding machine ko protect karna

Correct Answer: B

Explanation: Welding fixtures repeated aur accurate assembly ke liye components ko sahi jagah par hold karte hain, distortion bhi kam karte hain.

Q68. Workpiece ko clean aur dry rakhna kyu important hai fabrication se pehle?

A) Taaki welding machine achhi chale

B) Taaki marking clear dikhe aur contamination na ho

C) Taaki tool slip na ho

D) Taaki zyada heat lage

Correct Answer: B

Explanation: Clean aur dry surface par marking clear hoti hai, aur welding ke dauran contamination se bacha ja sakta hai.

# Module 5: Welding Defects & Quality Control

Q69. Weld mein 'Porosity' kya hai?

A) Weld bead ka bahut mota hona

B) Weld metal ke andar trapped gas bubbles

C) Weld ka bahut patla hona

D) Weld surface par cracks

Correct Answer: B

Explanation: Porosity weld metal mein gas bubbles ka trap ho jana hai, jo shielding ki kami ya contaminated material se hota hai.

Q70. 'Undercut' kya hai?

A) Weld metal ka base metal ke upar flow karna

B) Base metal mein ek groove ya notch jo weld toe ke paas banta hai

C) Weld metal ka joint se bahar nikalna

D) Weld ka bahut zyada penetrate karna

Correct Answer: B

Explanation: Undercut base metal mein ek notch hai jo weld toe ke paas banta hai, jisse strength kam hoti hai. High current ya wrong angle se hota hai.

Q71. 'Lack of Fusion' kya hai?

A) Do weld passes ka achhe se mix hona

B) Weld metal ka base metal ya previous weld pass ke saath properly fuse na hona

C) Weld ka bahut zyada fusion

D) Weld ka over-penetration

Correct Answer: B

Explanation: Lack of fusion tab hota hai jab weld metal base metal ya previous weld pass ke saath achhe se bond nahi ho pata, jisse weak spots bante hain.

Q72. Welding mein 'Distortion' ka kya matlab hai?

A) Weld bead ka colour change hona

B) Heat ke karan metal component ki shape ya dimension mein change aana

C) Weld metal ka hard ho jana

D) Weld ka bahut zyada smooth hona

Correct Answer: B

Explanation: Welding heat ke karan metal expand aur contract karta hai, jisse component ki original shape mein badlav aa sakta hai.

Q73. 'Slag Inclusion' kya hai?

A) Weld metal ka bahut clean hona

B) Non-metallic material (slag) ka weld metal ke andar trap ho jana

C) Weld surface par bumps

D) Weld ka bahut zyada chamakna

Correct Answer: B

Explanation: Slag inclusion tab hota hai jab slag weld metal mein trap ho jata hai, usually improper cleaning ya high travel speed ke karan.

Q74. Visual inspection mein kya check kiya jata hai?

A) Internal defects jaise cracks

B) Weld bead ka appearance, size, shape aur surface defects

C) Metal ki chemical composition

D) Weld ki hardness

Correct Answer: B

Explanation: Visual inspection mein weld surface par dikhne wale defects jaise undercut, porosity, crack, aur bead profile check kiye jate hain.

Q75. Excessive travel speed se kya defect ho sakta hai?

A) Too much penetration

B) Lack of penetration aur narrow weld bead

C) Wide weld bead

D) No spatter

Correct Answer: B

Explanation: Agar travel speed bahut zyada ho, toh weld metal ko proper penetration aur fusion ke liye enough time nahi milta.

Q76. Too low amperage se kya ho sakta hai?

A) Deep penetration

B) Lack of penetration, poor fusion aur electrode sticking

C) High spatter

D) Wide weld bead

Correct Answer: B

Explanation: Kam amperage se heat input kam hota hai, jisse weld metal properly melt nahi hota aur penetration kam rehti hai.

Q77. Welding mein 'Cracks' kyu aate hain?

A) Metal ka bahut soft hona

B) High carbon content, rapid cooling, ya excessive stress ke karan

C) Weld ka bahut zyada mota hona

D) Proper shielding gas use karna

Correct Answer: B

Explanation: Cracks high stress, inappropriate material, improper pre-heat/post-heat, ya rapid cooling ke karan aate hain.

Q78. Weld bead ki uniformity kyu zaroori hai?

A) Sirf appearance ke liye

B) Stress distribution aur strength maintain karne ke liye

C) Taaki welding faster ho

D) Taaki electrode kam use ho

Correct Answer: B

Explanation: Uniform weld bead stress ko evenly distribute karta hai aur joint ki strength ko maintain karta hai.

Q79. Welding ke baad 'spatter' kya hota hai?

A) Weld metal ka ek smooth surface

B) Chhote metal particles jo weld area ke aas-paas chipak jate hain

C) Weld metal ka colour change

D) Weld joint ki depth

Correct Answer: B

Explanation: Spatter molten metal ke chhote droplets hote hain jo arc se bahar nikal kar workpiece par ya aas-paas chipak jate hain.

Q80. Spatter ko kam karne ke liye kya kiya ja sakta hai?

A) High amperage use karna

B) Proper arc length, current setting aur electrode angle maintain karna

C) Thick electrode use karna

D) Fast travel speed

Correct Answer: B

Explanation: Correct welding parameters maintain karne se spatter ko kafi had tak control kiya ja sakta hai.

Q81. Welding mein 'Burn-through' kya hai?

A) Weld ka bahut shallow hona

B) Molten weld metal ka workpiece ke through pass ho jana, jisse hole ban jata hai

C) Weld ka bahut clean hona

D) Excessive slag formation

Correct Answer: B

Explanation: Burn-through tab hota hai jab high heat input ya thin material ke karan weld metal workpiece se poora nikal jata hai.

Q82. Welding defects ko avoid karne ke liye sabse important step kya hai?

A) Fast welding karna

B) Proper joint preparation aur correct welding parameters set karna

C) Bina PPE ke kaam karna

D) Kam heat use karna

Correct Answer: B

Explanation: Achhi joint preparation aur sahi welding parameters defects ko minimize karne ki key hain.

Q83. Weld bead ki convex (bahar ki taraf ubhri hui) shape kis baat ka indication ho sakti hai?

A) Perfect weld

B) Insufficient heat input ya too fast travel speed

C) Excessive penetration

D) Lack of fusion

Correct Answer: B

Explanation: Excessive convexity (over-reinforcement) usually insufficient heat input ya fast travel speed ke karan hoti hai, jisse weld profile achha nahi banta.

Q84. Visual inspection ke liye kaun sa tool use hota hai?

A) Micrometer

B) Welding gauge

C) X-ray machine

D) Hardness tester

Correct Answer: B

Explanation: Welding gauge weld bead size, fillet weld leg length, throat thickness, undercut aur misalignment jaise parameters ko measure karta hai.

# Module 6: Advanced Concepts & Workshop Management

Q85. MIG welding ka full form kya hai?

A) Manual Inert Gas

B) Metal Inert Gas

C) Machine Inert Gas

D) Molten Inert Gas

Correct Answer: B

Explanation: MIG welding ko Metal Inert Gas welding ya GMAW (Gas Metal Arc Welding) bhi kehte hain.

Q86. MIG welding mein shielding gas ka kya kaam hai?

A) Weld metal ko thanda karna

B) Atmospheric contamination se weld pool ko protect karna

C) Arc ko ignite karna

D) Wire feed speed control karna

Correct Answer: B

Explanation: Shielding gas oxygen aur nitrogen ko weld area se door rakhta hai, jisse weld defects nahi aate.

Q87. TIG welding ka full form kya hai?

A) Tungsten Inert Gas

B) Thin Inert Gas

C) Tool Inert Gas

D) Thermal Inert Gas

Correct Answer: A

Explanation: TIG welding ko Tungsten Inert Gas welding ya GTAW (Gas Tungsten Arc Welding) bhi kehte hain.

Q88. TIG welding mein kaun sa electrode use hota hai?

A) Consumable electrode

B) Non-consumable tungsten electrode

C) Carbon electrode

D) Copper electrode

Correct Answer: B

Explanation: TIG welding mein non-consumable tungsten electrode use hota hai, aur filler rod alag se feed kiya jata hai.

Q89. MIG welding ka ek major advantage kya hai?

A) Very slow welding speed

B) High deposition rate aur continuous wire feed

C) Only for thin materials

D) Produces a lot of slag

Correct Answer: B

Explanation: MIG welding high deposition rate aur continuous wire feed ke karan faster hai, jisse productivity badhti hai.

Q90. TIG welding ka primary advantage kya hai?

A) Very fast welding speed

B) High quality, precise welds aur no spatter/slag

C) Low skill required

D) Only for mild steel

Correct Answer: B

Explanation: TIG welding sabse clean aur precise welds deta hai, jo critical applications ke liye ideal hai.

Q91. Welding machine ke cables ko kaise maintain karna chahiye?

A) Unhe tight coil karke rakhna

B) Regularly inspect karna aur damaged cables ko replace karna

C) Unhe floor par chhod dena

D) Unhe pani mein dhona

Correct Answer: B

Explanation: Damaged cables electrical hazards create kar sakte hain aur poor weld performance de sakte hain.

Q92. Workshop mein tools ko organize karna kyu zaroori hai?

A) Taaki workshop bhara hua lage

B) Taaki tools easily mil sakein aur accidents kam ho

C) Taaki tools kharab ho jayein

D) Taaki kaam slow ho

Correct Answer: B

Explanation: Organized tools se efficiency badhti hai aur tripping hazards kam hote hain.

Q93. Gas cylinders ko kaise store karna chahiye?

A) Horizontal position mein

B) Vertically aur chain se secure karke

C) Direct sunlight mein

D) Kisi bhi jagah par

Correct Answer: B

Explanation: Gas cylinders ko vertically store karna chahiye aur chain se secure karna chahiye taaki woh gir na sakein.

Q94. Welding torch ke nozzles ko regularly clean karna kyu zaroori hai?

A) Taaki nozzle garam ho jaye

B) Taaki proper shielding gas flow aur arc stability maintain ho

C) Taaki nozzle shine kare

D) Taaki zyada spatter ho

Correct Answer: B

Explanation: Clogged nozzles shielding gas flow ko disrupt karte hain aur weld quality ko affect karte hain.

Q95. Basic blueprint reading mein welding symbol ka kya matlab hota hai?

A) Weld ki location aur type batana

B) Welder ka naam batana

C) Welding machine ka model batana

D) Welding ki speed batana

Correct Answer: A

Explanation: Welding symbols drawings par weld type, size, location aur other details ko graphically represent karte hain.

Q96. Welding workshop mein 'good housekeeping' ka kya matlab hai?

A) Sirf floor saaf rakhna

B) Regularly clean karna, tools ko organize karna aur hazards ko remove karna

C) Tools ko kahi bhi rakhna

D) Sirf welding machine ko saaf karna

Correct Answer: B

Explanation: Good housekeeping ek safe aur efficient working environment banata hai.

Q97. Welding machine ke cooling system ko kyu maintain karna chahiye?

A) Taaki machine zyada noise kare

B) Taaki machine overheat na ho aur uski life badhe

C) Taaki machine jaldi garam ho

D) Cooling system ka koi kaam nahi

Correct Answer: B

Explanation: Proper cooling machine ko overheating se bachata hai, jisse uski efficiency aur lifespan badhti hai.

Q98. Material handling ke liye kya precaution leni chahiye?

A) Heavy material ko akele uthana

B) Proper lifting techniques use karna ya lifting equipment ka sahara lena

C) Rough surface par material drag karna

D) Gloves nahi pehenna

Correct Answer: B

Explanation: Proper material handling techniques aur equipment se back injuries aur accidents se bacha ja sakta hai.

Q99. Welding ke liye 'Consumables' kya hote hain?

A) Welding machine

B) Shielding gas, electrodes, filler wires jo welding process mein consume ho jate hain

C) Safety glasses

D) Workpiece

Correct Answer: B

Explanation: Consumables woh materials hain jo welding process ke dauran use ho jate hain aur unhe regularly replace karna padta hai.

Q100. Ek welding technician ke liye continuous learning kyu zaroori hai?

A) Kyuki usko naye tools nahi sikhne

B) Industry mein naye processes aur materials aate rehte hain, skills ko up-to-date rakhne ke liye

C) Kyuki usko bas ek hi kaam karna hai

D) Kyuki usko sab kuch aata hai

Correct Answer: B

Explanation: Welding industry constantly evolve karti hai, isliye up-to-date rehne ke liye continuous learning essential hai.